The series DVR is designed specially for security and surveillance field which is an outstanding digital surveillance product. It introduces embedded LINUX operating system which is more stable. It introduces standard H.264mp video compressed format and G.711A audio compressed format which insures the high quality image and low error coding ratio. Strong network services greatly enhance the network data transmission capacity and remote control capabilities.
The First Part: Basic Operation

1 Basic Installation

1.1 Hard Disk Installation
For the first use，please install the hard disk. 

Full Function series DVR support 2 Sata hard disks, and Simple Function series DVR support 1 Sata hard disk.

Note: DVR can run and monitor normally without hard disk, while it can’t record and playback.

1.2 Mouse Connection

Full Function series DVR has 1 USB at back panel and 1 USB at front panel,the 2 USB could be used for mouse,Flash stick,3G&WIFI module,movable HDD and USB DVR-RW.
Simple Function series has two USB at back panel for mouse and flask stick
2 Turn on

Plug the power supply and turn on the power supply switch. Power supply indicator light shining indicates turning on the video recorder. After finishing the startup you will hear a beep, and the default setting of video output is multiple-window output mode.

Note: Restart the power after the abnormal power off, DVR will automatically recover the state before power off.

3 Turn off

There are two methods to turn off the DVR which are soft switch and hard switch. Entering [Main menu] and choosing [Logout] in the [turn off the system] option is called soft switch. Pressing the power supply switch is called hard switch.

Note: The setting information must be saved before replacing the battery otherwise information will lose.
4 Login

When the DVR boots up,user must login and the system provides the corresponding functions with the user purview. There are two user settings which are admin, and guest; and they have no passwords. “Admin” is the super user purview; “Guest” is the common user purview. 

Password protection: If the password is continuous wrong three times, the alarm will start. If the password is continuous wrong five times, the account will be locked. (Through reboot or after half an hour, the account will be unlocked automatically).
[image: image1.bmp]Picture1 Login 
For your system security, please modify your password after first login. 

5 Preview

Login normally and choose the multi-menu preview status.

The system date, time and channel name are shown in each viewing window. The surveillance video and the alarm status are shown in each window.
	1
	[image: image21.jpg]Ao e 1D X

MoveEye

Size:0x0.




	Recording status
	3
	[image: image2.bmp]
	Video loss

	2
	[image: image3.bmp]
	Motion detect
	4
	[image: image4.bmp]
	Camera lock


Table1 Preview icon

6 Recording Config

Set the recording parameters in the surveillance channel. You can enter [Main Menu]> [Record]> [Record Config] to set. The system’s default set is 24 hours continuous recording.

Note: There is at least one read-write hard disk if the DVR records normally.

[image: image5.png]/I.Main Menu->Record->Record Config

1 Redundancy (J
min Prerecord

Record Mode @ schedule O Manual O stop

Regular

Detect





Picture2 Record Config
【Channel】Choose the corresponding channel number to set the channel. Choose the “all” option to set the entire channels.  

【Redundancy】Choose the recording file and backup in two hard disks. The one is read-write disk, another is redundant disk. 

【Length】Set the time length of each video file. 60minutes is default length.

【Prerecord】Record 1-30 seconds before the action. (file length is decided by the code stream)

【Record Mode】Set video state: configuration, manual and stop.

Schedule：Record according to the set video type (regular, detect and alarm)and time section. 

Manual：Click the “all” button and make the corresponding channel recording no matter it is in any state.

Stop：Click the “stop” button and make the according channel stop recording no matter it is in any state.

【Period】Set the time section of common recording, The recording will start only in the set range.

【Recording Type】Set recording type: regular, detection or alarm.

regular：Perform the regular recording in the set time section. The video file type is “R”. 

detect：Trigger the “motion detect”, “camera mask” or “video loss” signal. When above alarm is set as opening recording, the “detection recording” state is on. The video file type is “M”. 

alarm：Trigger the external alarm signal in the set time section. When above alarm is set as opening recording, the “detection recording” state is on. The video file type is “A”. 
7.Snapshot Storage.(partial devices supported)
Setup snapshot parameters for different channels.At first time it's set for 24hours snapshot continuously, pls go to Main Menu->Record->Snapshot Storage for appropriate settings.

Note:If normal snapshot storage,pls setup Snap at MainMenu->Advanced->HDD Manage->Snapshot. 

[image: image13.png]/" SYSTEM LOGIN O\

User Name

Password

Please Login




Picture3  Snapshot Storage              
8 Video Playback 

There are two methods for you to play the video files in the hard disk.

1. In the desktop shortcut menu.

2. Main Menu>Record>Playback.

Note: The hard disk that saves the video files must be set as read-write or read-only state. 
[image: image14.jpg]® 0 EHmEEE




Picture4 Video Playback

1.File option
2.File information

3. File searching
4.File Backup
5. Operation hint      6.Playback control
【File option】Choose the file to playback/backup. 

【File information】Start time,end time and size.
Note: The storage must be large enough before the file backup.
【File searching】Search the file according to the searching parameter.

【File Backup】Backup files from HDD
【operation hint】Display the function of the cursor place.

【playback control】Refer to the following sheet for more information.

[image: image15.png]Wl Main Menu->Record->SnapShot Storage

Channel
Presnap
Record
Wweek
Period 1
Period 2
Period 3

Period 4

5

Pictures

@ schedule

Manual O stop

Thu

Regular

00 :

00 :

00 :

00 :

Detect




Table2 Playback control key
Note: Frame by frame playback is only performed in the "pause" playback state.
Special functions:
Accurate playback:Input time (h/m/s) in the time column and then click playback.

Local zoom:When the system is in single-window full-screen playback mode, you can drag your mouse in the screen to select a section and then left click mouse to realize local zoom. You can right click mouse to exit.

9 Network Setup

[image: image6.png]Main Menu->System->NetWork —

Net Card wire Netcard |7) O DHCP Enable

1P Address 192 168 . 1 . 10

Subnet Mask |55 . 255 . 255 . o0

Gateway 192 . 168

Primary DNS 0o . o

Secondary DNS [ 0 . o

TCP Port

HTTP Port 80

HS Download ()

Transfer Policy | Quality Prefer]





Picture5 Network Setup

Doing network setup through Main Menu>System>Network

【IP address】Set the IP address. Default: 192.168.1.10.

【Subnet Mask】Set the subnet mask code.     

 Default: 255.255.255.0.

【Gateway】Set the default gateway. Default: 192.168.1.1.

Make the equipment’s IP address and the router in the same network segment. For example, the router IP address is 192.168.1.1, and the subnet mask is 255.255.255.0. According to the default setup, generally the default gateway is the router IP address, so enter IP address 192.168.1.10 in the IE browser to achieve visiting LAN equipment such as operating image surveillance in the public network remote access equipment for monitoring. Detail setup refer to “Network setup reference”.

10 Alarm Setup

 [image: image7.png]4 7 Main Menu->Alarm->Motion Detect

Channel Enable 8@
Sensitivity Region

Period Interval Sec.

Alarm Output sec.
Record Channel [1)]

Tour

Snapshot @l

PTZ Activation

Show Message 8 send Email ()
Buzzer (]} FTP upload (J





Picture 6 Motion Detection

Motion detection setup. Set DVR equipment making it alarm, linkage record and make the PTZ turn to preset position when there is a moving object in designated areas during Monday to Friday.

Step1. Choose【Main Menu】>【Alarm】>【Motion Detect】，then enter the setup interface;

Step2. Choose one channel and adjust sensitivity level, then set the time period of alarm surveillance. Set the monitor area(click the “set” button, choose the areas you want, and the “default” means choose all areas, then righclick to choose "up window".

Step3. When at alarm state, the DVR can take linkage measures. Alarm output, recording, tour, PTZ activation, snapshot,show message, sending email and buzzer.

 Step4. Set other channels alarm parameter following step 2 and step 3.

Note: video block, video loss and alarm input’s setup method are similar with motion detection.

10 PTZ Control

Through the following steps doing easy PTZ operations.

Step1. Set basic parameter such as channel and protocol. 【Main Menu】>【System】>【PTZ Config】 

Step2. At the state of menu preview, enter the PTZ operation menu.

Step3.One is "rightclick" mouse, choose the “PTZ control” and doing the PTZ general operation. The other method is "rightclick" mouse, choose “High Speed PTZ”. Control the left button and move it to control the PTZ direction directly. Use the mouse wheel to zoom in/out the controlled camera.

The Second Part: Remote Control

1 Network Connection

Before web operation,you need to connect the equipment with internet.
Step1.Connect the equipment with the internet correctly.
Step2.Set LAN following to the ninth term Network Setup in  reference manuals first part.
Step3.when Lan is connected,it needs to set up port mapping for public network access.
Step4.DDNS Application.You can apply for DDNS in the http://www.3322.org/  and http://www.oray.com/
Step5.DDNS Setup.Make sure to fill the right info in the Main Menu->System->NetService->DDNS
Note:The device must be in the same network with PC,if cannot connect,pls check the device IP if it works fine.
2. Remote Monitoring

After connecting the network, two kinds of methods can be used for remote monitoring which are client software and common browser.Client software(CMS) is professional software used in multi-equipments monitoring, which is safe, convenient and stable. It is the better one that we advise customers use it. And the browser comes with system such as IE browser.

2.1 Client software
CMS software is used in a computer, and manipulates several DVR at the same time.

Step1. Please take CD out of the accessory box, then copy CMS installed software in it.

Step2.After installing this software at the local PC, then open the operation system as in picture 6,Enter the add device interface through【System】>【Device Manager】>【Add area/device】, and enter DVR information which is needed manipulating following the prompts, in that way, you can manipulate any specified equipment. And the method is the same with web manipulation’s. 

Step3. You can add several DVR more to do monitor operation if you repeat this operation.

[image: image8.png]oS, =& X

20:50:14
2010-08-04





Picture 7 CMS Interface

2.2 WEB

After the network connecting, using the browser comes with computer login DVR to do remote monitoring. 

Step1. install the plug. Open the disk, then get General_IE_V2.0.0.35_20111222.exe and install. 

Step2. Connect the equipment. Open web browser, then enter IP address on the address bar of the equipment. For example, the IP address of the equipment is 10.6.0.114, and then you should enter http://10.6.0.114 on the address bar to connect.

Pop up as follows interface after connecting successfully.

Note: when  first  login, see  below  picture
[image: image9.png]veo 2] e SEY LRE KPO IAD WM - O X

o = HE
€9 QUNH- B puomesnd FIEIE ) a
T T

O PR GEmER - M 5% @] BR O FiEKE G RRRE e o @ RS (- D - TE - LERES -

w IR

O[O BEEE RE ae

@D L0 Qo -





Picture 8 Web Login Interface

Step3. Login. Enter user’s name and password, the default Administrator User ID is admin, and the password is admin. The user should change the administrator’s password timely after loginning in.(If choose "Local Area Network",it's Main streams,if "wide Area Network",it's Sub streams.)
You will see as follows interface after loginning in.
[image: image16.jpg]StartTime Type
| Ed 165710 R

tart Time
0-08-04 1655710

nd Time 2—
04 16:5717

0000002000





Picture 9 WEB operation interface

3. Basic Web Operation 

Do web operation in picture 8 web operation interface

1) Screen Split

Choose image preview mode;

2) Playback

Enter playback mode, which supports multi-channel playing back simultaneously. 
3) Log

Display log information；

4) Local Config

Alarm and System setting
5) Channel operation
Open the videos. Left click and choose a window in the left, and choose the corresponding channel in the right, then double click the video. Open the second channel’s video, choose the left window, then select the corresponding channel in the right, after then, double click the video. Opening other channels is the same way. If you have chosen the window of opened video, then open other channels in the right, the system will close all the channels and open new channel. Customers can adjust image mode upon opened video channels.

Close the channel. Rightclick mouse at the channels which are needed to close. Or choose to close all the windows to close opened video channels.

The Third Part: Special Function

This series become more personalization design and visualization operation press key,local enlarge at any areas in preview interface,timing start up and shut down,FTP upload,3G&WIFI and so on.Now let's introduce Encode,Multi channels playback and mobile monitor setup.
1 Coding Config

DVR achieves the functions such as high quality playback and remote monitoring by coding parameter config.

[image: image17.png]Buton | Function Buiton | Function
oo, @ [pecat
2] | S ) | stowpiay
& [ | 5 |renrer
Y| Frame g [
G || [Fer
B |





Picture 10 Encode setup
1.1 Encode setup for every channel
Note: Only part of DVR mode support resolution switch.
Step1. Local operation through 【Main Menu】>【System】>【Encode】（remote setup click right mouse and choose “config” in【System】>【Encode】）.

Step2. Choose channel 1, set the resolution as D1 (25fps, 1536Kb/s)

Step3. Choose channel 2, set the resolution as CIF (25fps, 512Kb/s). Click right mouse or choose “Advanced” then click “Copy” button.

Step4. Choose channel 3, click the right button or choose the local “Advanced”, then choose “Stick”. Channel 4’s operation is the same with channel 3.

Step5. Click “save”, then exit.

Note: standard parameter between resolution and Kb/s.

	Resolution
	Kb/S

	D1
	512~2560

	HD1
	384~2048kbps

	CIF
	64~1024kbps

	QCIF
	64~512kbps


1.2 Extra Stream Setup

Extra stream is used for remote client monitor and mobile monitor area.

    Step1. Enable extra stream

Step2. Set frame rate and bit rates,operation way is the same as  main stream.
2 Multi-channel Playback Simultaneously

DVR introduces the newest combination coding technique to achieve videos of all channels play simultaneously.

4 channel DVR can support 4channels playback simultaneously,8 channel DVR can support 8channels playback simultaneously,channel number can be free chosen. 

[image: image10.jpg]StartTime Type
150216 R

IStart Time
10-08-05 15:02:16
End Time
10-08-05 15:02:38
ISize(KB)

4096

OOQQ@@@@GQ —





Picture11 Video Playback

Step1. Set every channel’s encode parameter by 【Main Menu】>【Record】>【Encode】.

Step2. Enter video playback interface, click Search button.

    Step3. Choose File Type and All and time period, then click OK in Search Condition interface.  

    Step4. Choose the record file, then click Play or double click the file to playback in playback interface.

3 Multi-channel Real-time Monitor remotely
Introduce extra-stream techniques to achieve multi-channel remote monitoring simultaneously in the condition of narrow band(poor network speed)
[image: image11.png]



 Picture12 CMS Interface

Step1. Enable extra streams(MainMenu->System->Encode)

Step2. Open CMS, and then login.

Step3.After add devices,rightclick device name to select "Connect all video(Main stream)"
4 Mobile Monitoring

It introduces extra stream techniques when mobile monitoring,support channels switch(eg.  Windows mobile)
     Step1. DVR config,go to  MainMenu->System->Encode->

Extra Stream,enable it.
 Step2. Setup mobile monitor, go to MainMenu->System->

NetService->Mobile Monitor,choose "Enable"

     Note: Pls map the mobile monitor port in the router first,then you can use mobile to visit it.

     Step3. Intall the corresponding mobile monitoring software through CD (Software->Mobile Monitor->MEYE->

MEYE_WM.cab)

     Step4. After successfully installing the software,finding and running "eg.windows mobile-moveeye" software through [start]->[procedure] in the mobile,see picture 14

[image: image18.jpg]k@) IAD EHO
o% GIKEE GRS

R fene 3922 ore

Al





Picture13 mobile client software Interface

Step5.Mobile setup.Click "setting" button at the right corner,then go to below picture 15. 

[image: image19.png]Fd Main Menu->System->Encode m

Channel 1

Compression H.264 Extra Streaml

Resolution DI(704x576) CIF(352x288)

Frame Rate(FPS) |25 12

Bit Rate Type  |VBR VBR

Quality Good General

Bit Rate(Kb/S)  |i536 107

1 Frame Interval

2 2
videojaudio @ O 2 0

(R (TR (e





picture 14  mobile client software setting

User name: login name of DVR

Password: password of DVR

Server: IP address or domain name of DVR

Port: mobile monitor port of DVR,pls refer to Step 2.

Auto Connect:If select it,then it will auto connect to the DVR and auto display cameras.

Note:DVR need to be connect to public net when mobile monitor
Similar setup between different operation system:

     Symbian S60 the 3rd operation system mobile monitoring client software: MEYE_SB_S60_3rd.sisx

     Symbian S60 the 5th operation system mobile monitoring client software: MEYE_SB_S60_5th.sisx

     Android mobile monitoring client software:MEYE_Android.cab

     Blackberry mobile monitoring client software:MEYE_RIM.cod

    Iphone mobile monitoring software: use mobile to search "vMeye" in the appstore and install it online.
5  Flexible File Storage and Backup

DVR introduces several unique storage and backup techniques to achieve multiple modes storage and backup.

Real-time storage

Redundant storage. DVR introduces RAD 1 storage technique to achieve two hard disks storing simultaneously and mutual backup.

USB HDD and movable hard disk. DVR introduces the storage technique, supporting video files which are real-time written in storage devices.

DVD-RW. DVR introduces the newest real-time storage technique, supporting video file which are burnt real-time to CD.

Real-time remote storage. DVR supports video files real-time storing to computer devices(Disk C/D/E/F) at the remote client.

File download

Using U disk and movable hard disk download at local device, DVR supports the chosen file are high-speed backup to the storage devices.

DVD-RW. DVR supports the chosen video file are burnt and stored as CD.

Network high-speed download. DVR supports high-speed down loading the chosen file at remote client.

Appendix 1. Remote controller operation

[image: image20.png]W THoveRy PN 1237 ok

Username: ~ [admin
Password:  [2008
server:  [test.3322.0rg
Port: 34508

Notes: =

Ok History

Auto Connect (O chinese

@ Engish

¥20100324-10





	Serial number
	Name
	Function

	1
	Multi-channels button
	Multi-channels preview

	2
	Number button
	Password input/number input/ channel switch

	3
	【Esc】
	Back to up window

	4
	Direction button
	Direction and OK button

	5
	Playback operate
	Playback Basic operation

	6
	Record control
	Enter into record menu

	7
	Remote controller
	Input the number of DVR to control it

	8
	FN
	Assistant function


[image: image12.jpg]® 0 EHmEEE




Appendix 2.Mouse operation

  *Take right hand as an example
	Operation
	Function

	Double left click 
	Double click one item in the file list to playback the video 
Double click the playback video to zoom in or out the screen 

	
	Double click the channel to make it full screen display in preview

double click again to resume the multi-channel display 

	Left click
	Choose according option in the menu 

	Right click
	Pop desktop shortcut menu in preview state 

	
	Current shortcut menu in the menu 

	Wheel button
	Add or subtract number value in the number setting

	
	Switch the items in the combo box 

	
	Page up or down in the list 

	Move mouse
	 Choose the widget or move the item in the widget 

	Drag mouse
	Set the motion detect area

	
	Set the cover area 


PRODUCT WARRANTY CARD

All the products sold are covered by 12months warranty from the date of invoice.

Warranty instructions：

1. Please contact us when the product is caused by its own fault within 1year warranty.

2. Please mail us your written warranty card as soon as possible after purchasing our products so that we can repair or replace this product to its original operation condition for free. Or the company will not deal with it.

3. Please write the truth information on the warranty card.

4. Paid for repairing as follows:

A: Equipment failure caused by human operation      

B: Equipment failure caused by not conforming to the using environment      

C: No warranty card     

D: Warranty expired 

Warranty card information:

Product model              Product serial number           Purchasing time                                        
User name                 Contact person                          Telephone                                             
Distributor Name           Contact person                            Telephone                                              
Note：Please keep the warranty card for the better service.

